March 9, 1981

Notes on Ithaca Intersystems' Relocating,

Extended Memory Test for the MPU-80

Features of "MEM.COM"

---------------------

This memory  test will work with or  without the MPU-80's windows

enabled.   Of course,  extended memory  (64 K-bytes  and up)  can't be

tested if  the windows are  disabled.  If both  4K windows are enabled

(enabling  only  one  4K  window  is  not  allowed),  the memtest will

automatically find it and display its address.  If it finds the window

and  its I/O  base address  isn't E0  hex (the  standard Intersystems'

configuration), then it  will ask you for the  correct MPU-80 I/O base

address.

When  the actual  memory test  is initiated,  the first  thing it

checks for  is address shorts.  If  the address lines are  OK, then it

proceeds to the non-M1 test which  checks each bit of every byte to be

tested.  If there  are still no errors and you  didn't tell it to skip

the M1  test, then it  will write a  pattern of two  Z-80 no operation

codes and  one restart 7  op-code throughout tested  memory (two zeros

and one FF  hex).  Then the cpu  jumps to the mess  and returns to the

memtest every  third byte  to check  all the  registers and the stack.

The purpose  of  this  M1  test  is  to  check  the  memory board's M1

look-ahead circuitry  or, if this doesn't exist,  it checks the memory

board's access  time, because  Z-80 M1  cycles allow  less access time

than non-M1  cycles.  Of course, the  ultimate test is to  run a large

program like  Pascal while the  memory board in  question is addressed

within 64K page 0.

Z-80 Memory Test                                             Page    2

How to use "MEM.COM"

--------------------

If the memory board you want to  test is known to be bad or it is

right out of production, then I  suggest you address it up in extended

memory.  Of  course, you'll need  to enable the  windows on the MPU-80

and the  extended  addressing  feature  on  all  memory  boards in the

system.  You'll also need at least 8K of memory starting at location 0

and enough additional memory to support the operating system, wherever

it may  be.  If  you are  using a  non-Intersystems' bios, remember to

either disable  the windows when  you are through  with the memtest or

modify the bios  so that it sets the windows  upon themselves during a

cold boot, otherwise you may have problems with other programs.  Also,

the  windows must  not be  addressed at  location 0  (A000 hex  is the

Intersystems' standard).

After running  "MEM.COM", a sign on message  should appear at the

console and if  the  windows  are  enabled,  the  address of the first

window will  be displayed.  If everything  is OK, it will  ask you for

the  OS size  in K  which will  default to  64 if  you just  give it a

carriage return.  If  you are using our cache BIOS  with a VIO or 6SIO

as the system I/O board, enter an 'X'.  The next question asks whether

or not you want to test M1 cycles.  This defaults to yes.

The start  and end  addresses are  processed together  and can be

specified in hexadecimal or K-bytes by using an "H" or "K" suffix.  If

both addresses  lack  a  suffix,  then  hexadecimal  is assumed.  If a

suffix is used on one address  and the other has no suffix, the result

is the same as using that suffix on both addresses.

The start  and end addresses are inclusive.   For example, if you

wanted to  test a 64K  board addressed for  64K page 0,  start and end

addresses of "0" and "FFFFH"  or "0" and "63K", respectively, would do

the job.

This  memtest will  also take  parameters from  the command line.

The first parameter must be either the  OS size in K or whether or not

you want to test M1 cycles.  That  is, after typing "MEM " to the CCP,

the first space must be followed  by a decimal number between 9 and 65

(or an  'X' for  our extended  memory cache  BIOS), a  "Y", an "N", or

another space  which  will  cause  both  parameters  to  take on their

default values.  If the first parameter  is the OS size in K, then the

second parameter can be either a "Y" or "N" to indicate whether or not

you want to  test M1 cycles or the start  and end addresses, otherwise

the second  parameter must be  the start and  end addresses.  Finally,

the last parameter accepted is  the number of passes desired which can

not exceed  1023 decimal.  If this parameter  is omitted, this memtest

goes on forever  (unless there is an address line  short or you hit an

"R" or a control C).  For  example, to completely test a 64K board (M1

cycles  and all), five  times, addressed for  64K page 0,  using a 64K

operating system, the following command lines would all work:

A>MEM 64 Y 0 63K 5

A>MEM 64 0 63K 5

A>MEM Y 0 63K 5

A>MEM  0 63K 5

Z-80 Memory Test                                             Page    3

If  you  have  any  questions,  suggestions,  complaints, bugs to

report, etc.,  please  refer  them  to  our  friendly customer service

department.

G.J.G.

Software Technician

##############################################################################################################################################################################################################################################################